# Configuring the Lane-Change Alert (PLCA)

How to turn on and adjust the lane-change alert without rebuilding the app

The EW8 display lets you add a brand-new alert by editing plain configuration files and dropping a few files next to the application – no programming and no rebuilding. The program file (nquick) stays exactly the same before and after. This guide uses the lane-change alert (also called PLCA) as a worked example, but the same four steps add any new signal-driven alert.

# What this feature does

The lane-change alert shows a coloured bar on the left or right edge of the screen when another vehicle is in your blind spot on that side:

A yellow bar means “information” – a vehicle is there.

A red bar means “warning” – don’t change lanes now.

The left and right sides are independent, so you can see, for example, a red bar on the left and a yellow bar on the right at the same time. If both yellow and red are sent for the same side, red is shown (it is the stronger warning).

# The idea: a new alert by configuration only

The display is driven by a set of text files and images that sit next to the application. To add an alert you change four things – no source code is touched:

A bus description file (.dbc) – tells the app how to read the alert message coming in over the vehicle bus.

The signals file – gives each indicator a name and connects the bus message to it.

One or more images – the picture(s) the alert shows.

The scene file – says where on the screen the alert appears.

When the app starts, it reads these files and creates the alert for you. A new name in the signals file becomes a brand-new alert automatically – nothing is fixed in the program in advance.

# What kinds of alerts you can add this way

Anything that is “a signal from the bus turns a picture (or a number) on the screen” can be added with configuration only. That includes:

An icon or bar that simply appears and disappears (like the lane-change bars).

An icon that blinks.

An icon that shows for a few seconds and then hides by itself (a timed sign).

A number shown on the screen (for example a distance or a speed).

A small animation (an animated image / GIF).

What is not configuration-only: an alert that needs brand-new on-screen behaviour that none of the above can do (special moving graphics or decision logic of its own). That kind of alert needs a developer. For everyday “show this when the bus says so” alerts, configuration is all you need.

# How to apply your changes (how it takes effect)

All of these files live in the application folder. You edit them with any text editor (Notepad, VS Code, etc.) – they are plain text – and copy the images and the .dbc into their folders. Then:

Make your edits / copy your files in (see the steps below).

Save.

Restart the application.

That’s it. The change appears the next time the app starts. The app reads these files fresh on every start-up, so there is nothing to compile and no developer needed. The same program file is used before and after – only the configuration around it changed.

# Where the files live

Next to the application you will find these folders:

DBC/        → bus description files (.dbc)

signals/    → EW8_Signals.json

configs/    → scene.json

assets/     → images

The app finds a .dbc automatically by its name, so simply putting the file in the DBC folder is enough – no list to update.

# Step 1 – The bus description file (.dbc)

This file tells the app which message on the bus carries the alert and which bit means what. Create a file in the DBC folder named after the protocol, for example:

DBC/JunctionBox_Protocol_led257.dbc

Inside, the important part describes one message and four on/off bits:

BO_ 768 JunctionBox_LCA: 8 Vector__XXX

SG_ LCA_Left_Info  : 0|1@1+ (1,0) [0|1] "" Vector__XXX

SG_ LCA_Right_Info : 1|1@1+ (1,0) [0|1] "" Vector__XXX

SG_ LCA_Left_Warn  : 2|1@1+ (1,0) [0|1] "" Vector__XXX

SG_ LCA_Right_Warn : 3|1@1+ (1,0) [0|1] "" Vector__XXX

In plain words:

768 is the message number (the same as 0x300).

Each SG_ line is one on/off flag: left/right, info (yellow) or warn (red).

Important: the message number and the bit positions must match what your vehicle / test equipment actually sends. Use the numbers from your real lane-change device; the values above are an example.

# Step 2 – Name the indicators and link them

Open:

signals/EW8_Signals.json

First, add four names so the display knows these indicators exist (put them near the other ALERT_ names at the top of the file):

"ALERT_LEFT_LCAI"

"ALERT_RIGHT_LCAI"

"ALERT_LEFT_LCAW"

"ALERT_RIGHT_LCAW"

(LCAI = info/yellow, LCAW = warn/red.)

Then, lower down, add a block that connects each bus flag from Step 1 to the matching indicator name:

{

"protocol": "JunctionBox_Protocol_led257",

"signals": [

{"name": "LCA_Left_Info",  "action": "ALERT_LEFT_LCAI",  "type": "GraphicItem"}

,{"name": "LCA_Right_Info", "action": "ALERT_RIGHT_LCAI", "type": "GraphicItem"}

,{"name": "LCA_Left_Warn",  "action": "ALERT_LEFT_LCAW",  "type": "GraphicItem"}

,{"name": "LCA_Right_Warn", "action": "ALERT_RIGHT_LCAW", "type": "GraphicItem"}

]

}

The “protocol” name must match the .dbc file name from Step 1. “name” is the bus flag; “action” is the indicator it turns on.

# Step 3 – The images

Put the two bar pictures here:

assets/images/LCA/LCA_Yellow.png   (the info bar)

assets/images/LCA/LCA_Red.png      (the warning bar)

You can replace these with your own artwork at any time – just keep the same file names, or update the names in Step 4 to match.

# Step 4 – Where the bars appear on screen

Open:

configs/scene.json

Add this block inside the “nodes” list:

{

"parent": "mainPanel",

"type": "group",

"id": "lcaPanel",

"layer": 1,

"children": [

{ "type": "image", "graphic_item": "ALERT_LEFT_LCAI",  "src": "A:images/LCA/LCA_Yellow.png", "align": "left_mid",  "offset": [2, 0],  "layer": 0 },

{ "type": "image", "graphic_item": "ALERT_RIGHT_LCAI", "src": "A:images/LCA/LCA_Yellow.png", "align": "right_mid", "offset": [-2, 0], "layer": 0 },

{ "type": "image", "graphic_item": "ALERT_LEFT_LCAW",  "src": "A:images/LCA/LCA_Red.png",    "align": "left_mid",  "offset": [2, 0],  "layer": 0 },

{ "type": "image", "graphic_item": "ALERT_RIGHT_LCAW", "src": "A:images/LCA/LCA_Red.png",    "align": "right_mid", "offset": [-2, 0], "layer": 0 }

]

}

What the values mean:

graphic_item – the indicator name from Step 2 (this links the picture to the bus flag).

src – which picture to show. “A:” means the assets folder.

align – left_mid puts it on the left edge, right_mid on the right edge.

offset – a small nudge in pixels [left-right, up-down] to fine-tune the position.

Tip: list the red (warn) bars after the yellow (info) bars, as shown. That way, if both arrive for the same side, the red one is drawn on top.

# How to check it works

Restart the app, then have your bus tool send the message (number 768 / 0x300) with different bytes. The first byte controls the bars:

300#06  →  left red + right yellow

300#0C  →  both sides red

300#00  →  all bars off

If nothing appears, re-check that the message number and bits in the .dbc match what your tool is sending, and that the four names match exactly in all files.

# Common adjustments

Move a bar inward/outward: change the first number in offset (left-right).

Move a bar up/down: change the second number in offset.

Use different artwork: replace the PNG files, or point src at a different file name.

Change which bus message/bits are used: edit the .dbc in Step 1.

Remember: after any change, just save and restart the app. No rebuild is ever required for these adjustments.